Project 7/8

Tom Heijmans (1079471)

Sandra Hekkelman &

Wouter Volders

Eerste gelegenheid

7/6/2026

Object Detection Model

Literature Research Report

-

Table of Contents

Summary	3

Introduction	3

Theoretical Framework	3

Methodology	3

Results	5

Conclusion	6

Glossary	7

Sources	7

Changelog	7

## Summary

This report investigates which object detection model is most applicable in an autonomous driving vehicle. For this vehicle to drive autonomously it must be able to detect different signs, people, other vehicles, road markings and traffic lights and differentiate between them. For that reason there must be an object detection model which is able to detect these objects with accuracy, low latency, and low computational load, whilst also not being too complex to train.

The comparison of YOLOv8, YOLO26, SSD and Faster R-CNN is based on the following factors:

- Latency; the model has to work mostly in real time (<= 1 second). The kart drives continuously in a dynamic environment and must react to obstacles in real time

- Accuracy; the model has to be able to reliably detect objects and has to work on smaller/further objects.

- Load; the model has to remain lightweight. More computationally intensive models increase CPU usage, which can cause frame-skipping, which in turn can cause missed detections. Load in this context stands for computational load.

- Trainability; since the model has to detect

  - signs (stop, speed, prohibited, directional (one-way), mandatory),

  - pedestrians,

  - vehicles (cars),

  - traffic lights.

The model has to at least be able to detect these object types and their subcategories. Therefore, the model must support simple and efficient training on custom datasets

The outcome of this research is that a YOLO-based model is recommended, based on the fact that YOLO models have the best latency and accuracy trade-offs, relatively medium computational load on the system, and simple trainability compared to the SSD and Faster R-CNN models.

## Introduction

This project focuses on the development of software to make a small kart drive autonomously. This is for the SDC (Self-Driving-Challenge) provided by the RDW (Rijksdienst voor het Wegverkeer) to give them more insight into how self-driving vehicles work, since most of the car companies developing this software do not want to publicly disclose their development procedures.

To have the kart drive autonomously it must be able to detect objects and obstacles that are on the road ahead of it, these objects include: people, cars, traffic signs and traffic lights. The detection will be done through cameras and an object detection model, but then the question is: Which object detection model is the most applicable for the described situation? This report investigates which object detection model is most suitable for the described application.

## Theoretical Framework

This report focuses on object detection models. Object detection models are already widely used in the field of autonomous vehicles [1]. Object detection models are an application of computer vision [1]. Computer vision is a field of artificial intelligence that trains machines to detect and extract meaningful information from digital imagery and videos [1]. Computer vision primarily relies on image processing, neural networks, and key tasks.

Image processing is a technique which enhances and cleans raw visual data so algorithms can analyze key features like edges, colors and textures.

Neural networks are computing systems inspired by the human brain [8]. They use deep learning to recognize complex patterns and features. These neural networks are commonly implemented in the form of object detection models, that are trained on millions of labeled images to learn how to detect and display these specific elements. CNN (Convolutional Neural Network)  is a type of neural network that learns features through filter optimization [8].

Key tasks handle processes like the detection of objects, the classification of these objects and the segmentation (outlining of the detected pixels) of these objects.

Most of the models that are chosen to be analyzed and compared in this report are either one-stage or two-stage detectors. The difference between one-stage detectors and two-stage detectors is that whilst one-stage detectors use a CNN (Convolutional Neural Network) to directly predict the bounding box coordinates and class probabilities from the input images in one pass, Two-stage detectors split the task into two steps: region proposal and region classification/refinement. Region proposal identifies potential areas and separates these whilst region classification classifies and tightens these bounding boxes for a higher accuracy [7].

To summarize,  being a one-stage detector means that it is faster and associated with a lower computational load [7], but less accurate. Being a two-stage detector means that it is more accurate, but slower, and a higher computational load [7].

Although object detection models are widely used in autonomous vehicles, relatively little research focuses specifically on selecting lightweight models while simultaneously balancing latency, accuracy and computational load.

## Methodology

The research method of this report is literature research. That means that this report is based on sources of the documentation of various different model types.

There are some requirements for choosing the right/most applicable detection model:

- Latency; the model has to work mostly in real time (<= 1 second). This is because the kart is constantly driving, and has to react to things that happen in time

- Accuracy; the model has to be able to reliably detect objects and has to work on smaller/further objects.

- Load; the model has to remain lightweight. The heavier models increase CPU usage, which can cause frame-skipping, which in turn can cause missed detections. Load in this context stands for computational load.

- Trainability; since the model has to detect

  - signs (stop, speed, prohibited, directional (one-way), mandatory),

  - pedestrians,

  - vehicles (cars),

  - traffic lights.

The model has to at least be able to detect these object types and their subcategories. Therefore, it is important for the model to be trainable.

Four models were selected for comparison: Yolov8, YOLO26, SSD and Faster R-CNN. These models were selected because they are among the most commonly used object detection architectures in real-time applications [1]. These models also provide a comparison between one-stage detection and two-stage detection.

A weighing table is also included aside from the main comparison table, the weighing table will include the scoring system used in the main table and is made because some of the previously described criteria are more important than others, for example, lower latency is more important than a simple training method.

The comparison between the selected models is based on data collected from official model documentation, benchmark reports and scientific publications [2]–[6]. For each model, information regarding latency, accuracy, computational load and trainability was collected and compared. These criteria were selected because they directly influence the suitability of an object detection model for use in an autonomous vehicle. The collected data were subsequently evaluated using the weighting system shown in Table 2, allowing the models to be compared according to the relative importance of each criterion.

The weighting system was developed based on the requirements of the autonomous kart. Latency and accuracy were assigned a higher priority because the system must detect and react to objects in real time while maintaining reliable detection performance. Computational load was also considered important due to the limited processing resources available on the target platform. Trainability was included as a criterion because the selected model must be capable of being trained on custom datasets containing traffic signs, vehicles, pedestrians and traffic lights.

## Results

In this research report a comparison is made between models from documents and datasheets of these models. The compared models are the YOLOv8, YOLO26, SSD and Faster R-CNN models [1]. These models were chosen based on that they are commonly used object detection models. Based on the requirements defined in the Method section, the object detection models are compared using the information gathered from official documentation, benchmark reports and research papers.

This comparison is mainly focused on four factors: Latency, Accuracy, Load and Trainability. The results of this comparison are presented in Tables 1 and 2 .

Table (1) – Data for models

Table (2) – Scoring System

Table 1 contains data obtained from model documentation, benchmark reports and research papers [2]--[6]. The ‘Weighting’ column in Table 1 is based on table 2. Table 2 has the Scoring System that is used for setting priorities between the different criteria. These priorities were made based on what I concluded to be the most important when it comes to choosing a good model.

The results indicate that all of the models are capable of real-time detection (under 1000 milliseconds).

However the two YOLO models show the lowest numbers of latency, which makes them suitable for applications where fast detection is required. The YOLO models are one-stage detectors and have a relatively high accuracy and low load on the system [2][3]. The simple trainability of the YOLO models is also beneficial for the application.

The SSD model, like the YOLO models, is also a one-stage detector [5]. It has a relatively low latency, whilst also having a lower accuracy, and a higher load on the system. The trainability is also simpler.

The Faster R-CNN model is a two-stage detector [6][7]. It has the best accuracy, but has a significantly higher latency and load on the system, whilst also having a complex method of trainability, which makes it less suitable for the described application.

Overall, the results indicate that one of the YOLO based models provide the best balance between latency, accuracy, load, and trainability for the set requirements defined in the method of this research. Whilst the SSD model has a lower accuracy and the Faster R-CNN model has a very high latency, compared to the other 3 models.

## Conclusion

To answer the question that was asked in the introduction, which was “Which object detection model is most applicable for the described application?”, from the results of this report, it can be concluded that a YOLO-based model is recommended to be used. This is because it fits the defined requirements. It has a low latency whilst providing a decent accuracy. It also has a relatively low load, and a simple trainability.

SSD has a relatively low latency, but also has a lower accuracy. Since the model must reliably detect objects, failure to detect critical objects may negatively affect the safety of the autonomous system. Therefore, the SSD model was considered less suitable for the described application

The Faster R-CNN model is more accurate, but this comes at the (very high) cost of latency. Whilst it is better to have it detect objects and obstacles more accurately, this reduces its suitability for real-time applications. The load is also higher than the YOLO models, which further reduces its suitability for the target system.

## Glossary

mAP		Mean Average Precision. Shows how precise the model is on average (in %).

Latency	The Delay between the time that the object is shown and actually detected.

Computational load	The amount of processing power something takes to run.

YOLO		You Only Look Once, a type of object detection model

## Sources

[1] Encord, "Object Detection: Models, Architectures and Techniques," Encord. [Online]. Available: https://encord.com/blog/object-detection/. [Accessed: 7-Jun-2026].

[2] Ultralytics, "YOLOv8 Documentation," Ultralytics. [Online]. Available: https://docs.ultralytics.com/models/yolov8/. [Accessed: 7-Jun-2026].

[3] G. Jocher et al., "YOLOv8: A State-of-the-Art Real-Time Object Detection Model," arXiv:2305.09972, 2023. [Online]. Available: https://arxiv.org/pdf/2305.09972. [Accessed: 7-Jun-2026].

[4] Ultralytics, "Object Detection Models Documentation," Ultralytics. [Online]. Available: https://docs.ultralytics.com/tasks/detect/#models. [Accessed: 7-Jun-2026].

[5] W. Liu et al., "SSD: Single Shot MultiBox Detector," in European Conference on Computer Vision (ECCV), 2016. [Online]. Available: https://arxiv.org/pdf/1512.02325. [Accessed: 7-Jun-2026].

[6] TensorBlue, "Object Detection: YOLO vs Faster R-CNN for Real-Time Detection," TensorBlue. [Online]. Available: https://tensorblue.com/blog/object-detection-yolo-faster-rcnn-real-time-detection-2025. [Accessed: 7-Jun-2026].

[7] Ultralytics, "What Is R-CNN? A Quick Overview," Ultralytics. [Online]. Available: https://www.ultralytics.com/blog/what-is-r-cnn-a-quick-overview. [Accessed: 7-Jun-2026].

[8] IBM, "Convolutional Neural Networks (CNNs)," IBM Think. [Online]. Available: https://www.ibm.com/think/topics/convolutional-neural-networks. [Accessed: 7-Jun-2026].

## Changelog

| Version | Changes | Date |
| --- | --- | --- |
| 0.1 | Document Made | 19-2-2026 |
| 1.0 | Document Version 1 Written and Finished | 13-3-2026 |
| 2.0 | Document Changed based on Feedback Thomas | 2-6-2026 |
| 2.1 | Document Finalized | 7-6-2026 |